🌍 EcoSort: AI-Powered IoT Waste Classifier

EcoSort is an intelligent waste management system built on MicroPython. By combining computer vision, the Google Gemini API, and precision robotics, it automatically identifies and segregates refuse into three streams: Recyclable, Non-Recyclable, and Organic.

✨ Core Capabilities

- Intelligent Sensing: Instant object detection via infrared (IR) sensors.

- Vision-Based AI: Captures high-resolution imagery and leverages Gemini's multimodal capabilities for high-accuracy sorting.

- Automated Logistics: A motorized sorting tray system driven by servos and stepper motors.

- Interactive Feedback: Real-time data visualization on a 20x4 LCD and dynamic status lighting via NeoPixel LEDs.

- Cloud-Enabled: Integrated WiFi for remote telemetry and over-the-air updates.

🛠️ Hardware Components

- ESP32 Microcontroller

- Camera Module

- IR Sensor

- 4 Servo Motors

- Stepper Motor

- 16x NeoPixel LED Ring

- 4x20 LCD Display

- WiFi Module

🔩 Hardware Architecture

| Component | Pin Number | Description |
| --- | --- | --- |
| Servo Motors |  |  |
| Front Servo 1 | GPIO 9 | Front sorting mechanism |
| Front Servo 2 | GPIO 8 | Front sorting mechanism |
| Drop Servo 1 | GPIO 7 | Drop mechanism |
| Drop Servo 2 | GPIO 44 | Drop mechanism |
| Sensors |  |  |
| IR Sensor | GPIO 41 | Object detection |
| LEDs |  |  |
| NeoPixel Ring | GPIO 1 | 16 LEDs for status indication |
| Stepper Motor |  |  |
| Stepper Motor | GPIO 2, 3, 4, 43 | Half-step motor control |
| LCD Display |  |  |
| I2C SDA | GPIO 5 | I2C data line |
| I2C SCL | GPIO 6 | I2C clock line |
| Camera |  |  |
| Camera | Default pins | OV2640 camera module |

Technical Note: Servos operate on a 50Hz PWM frequency. The LCD is pre-configured to I2C address 0x27.

📋 Prerequisites

- MicroPython firmware

- Python 3.x

- ESP32 development board

- Required hardware components

- WiFi network access

- Google Gemini API key

⚙️ Setup & Deployment

1. Firmware Initialization

Prepare your ESP32 by flashing the latest MicroPython binary:

Bash

esptool.py --port /dev/ttyUSB0 erase_flash

esptool.py --port /dev/ttyUSB0 --baud 460800 write_flash -z 0x1000 micropython.bin

2. Library Dependencies

Access the MicroPython REPL and install the necessary networking library:

Python

import upip

upip.install('urequests')

3. Configuration

Update Garbage-Separator.py with your local network details and API credentials:

- SSID: Your network name

- PASSWORD: Your WiFi security key

- GEMINI_API_KEY: Your unique Google AI Studio key

🛠️ Operational Workflow

- Boot-up: The device initializes and establishes a WiFi handshake.

- Detection: An object is placed in the hopper, triggering the IR Sensor.

- Analysis: The camera captures a frame; the image is sent to the Gemini API for classification.

- Sorting: Based on the AI's response, the Stepper and Servos align the chutes.

- Completion: The item is deposited, and the UI resets for the next cycle.

📊 Taxonomy of Waste

The system is calibrated to recognize and sort into three primary categories:

- ♻️ Recyclable: Metals, glass containers, clean paper, and HDPE/PET plastics.

- 🗑️ Non-Recyclable: Composite materials, contaminated packaging, and residuals.

- 🍎 Organic: Food scraps, garden waste, and compostable matter.